CHAPTER I

INTRODUCTION

1.1	General

Kerala regionally referred to as "Keralam'', better known as God’s own country is today one of the most sought after tourist destinations in India. Spread over 38,863 sq.km, it is bordered by Karnataka to the north east, Tamilnadu to the east and south, and the Lakshadweep Sea to the west. With 33,387,677 inhabitants as per the 2011 census, Kerala is twelfth largest state by population and is divided into 14 districts. The state has a coast of 590km and the width of the state varies between 11 and 121km. Geographically, Kerala can be divided into three climatically distinct regions: the eastern highlands, rugged and cool mountainous terrain with tea, coffee, rubber and spice plantations and wildlife reserves, the central midlands; rolling hills rich with cashew, coconut, arecanut, tapioca, banana, rice, ginger, pepper, sugarcane and vegetable plantations, and the western lowlands; networked by endless backwaters and the deltas of forty-four rivers serving as a drainage basin conveying a huge monsoon run-off into a vast inland water body stretching along the coastline.

Kerala is having very good network of various transportation modes like roadways, railways, airways and waterways. Kerala has 145,704kms of roads; it translates to about 4.62kms of road per thousand population, compared to an average of 2.59km in the country. There are 8 National Highways (NH) that connects all the major cities and towns within the state and outside having length of 1,524kms. There are almost 50 State Highways which connect every corner of the state having length of 4341.6kms and 18,900kms of district roads. Road traffic in Kerala has been growing at a rate of 10 - 11% every year, resulting in high traffic and pressure on the roads. Road density is nearly four times the national average, reflecting the state's high population density.

The rails run through the state connecting most of the major towns and cities except those in the highland districts of Idukki and Wayanad. The total length of the rail route is 1050km which includes 13 major railway routes with 200 railway stations in the state.

Kerala has three international airports; Trivandrum International Airport, Cochin International Airport and Calicut International Airport. It connects almost all the parts of country and abroad. Cochin International Airport is the largest and busiest in the state, and was the first Indian airport to be incorporated as a public limited company; it was funded by nearly 10,000 non-resident Indians from 30 different countries.

Water transport system is free of pollution, accidents and is affordable. As Kerala has numerous backwaters, waterways are used for commercial inland navigation. The transportation is mainly done with country craft and passenger vessels. There are 67 navigable routes in the state. The total length of the waterways in the state is 1895kms. This includes navigable river, backwaters and man-made cross canals. Most of these are in Travancore -Cochin region. Of the 44 rivers in Kerala, the 41 west flowing rivers together with backwaters and man- made canals form the integral part of inland navigation system. The main constraints to the expansion of inland navigation are lack of depth in the waterway caused by silting, lack of maintenance of navigation system and bank protection, accelerated growth of the water hyacinth, lack of modern inland craft terminals, and lack of a cargo handling system.

The main arterial Waterway in the state is the West Coast Canal. The West Coast Canal connects the Nileswaram in the North to Kovalam in the South and is about 590km. including the 47km. uncut portion from Azhikkal to Badakara. The West coast canal runs almost parallel to the sea and National Highway. A portion of the West coast canal, i.e. from Kottappuram to Kollam, is already declared as National waterway-III in 1993. It is the first national waterway in the country with 24 hours navigational facilities along the entire stretch. Kottappuram- Kollam route is almost in the middle of the WCC , based on that the WCC can be broadly divided into three sections namely Nileswaram to Kottappuram (348km), Kottapuram – Kollam (168 km) and Kollam-Kovalam 74km respectively.

1.2	Importance of Inland Water Transport (IWT)

IWT can play a significant role in the regions where the availability of road and rail systems is inadequate. One of the major advantages of water transport is that waterways being the gift of nature, and can be used for navigation with minimum investment. The benefit of water transport lies in its capacity to move bulk cargo at a cheaper cost. The surface transport infrastructure has reached the limit of their capacities and is failing to adequately respond to the ever-increasing traffic demand. Augmenting the capacities of road and rail system require huge investment and long gestation period.

1.3	National Waterways in India

Five waterways have been declared as National Waterways and another 101 are undergoing feasibility studies. The already developed National Waterways are:

•	National Waterway-1: Allahabad–Haldia stretch of the Ganges–Bhagirathi–Hooghly river of total length 1620 km was declared as National Waterway-1 (NW-1) in the year 1986.

•	National Waterway-2: Sadiya–Dhubri stretch of the Brahmaputra river of total length 891 km was declared as National Waterway-2 (NW-2) in the year 1988.

•	National Waterway-3: Kollam–Kottapuram stretch of West Coast Canal and Champakara and Udyogmandal canals of total length 205 km was declared as National Waterway-3 (NW-3) in the year 1993.

•	National Waterway- 4: Kakinada–Pondicherry stretch of canals and Kaluvelly tank, Bhadrachalam–Rajahmundry stretch of River Godavari and Wazirabad–Vijayawada stretch of River Krishna of total length 1095 km was declared as National Waterway-4 (NW-4) in the year 2008.

•	National Waterway-5: Talcher–Dhamra stretch, Geonkhali–Charbatia stretch of East Coast Canal, Charbatia–Dhamra stretch of Matai river and Mahanadi delta rivers of total length 620 km was declared as National Waterway-5 (NW-5) in the year 2008

India has extensive network of rivers, lakes and canals, which if developed for shipping and navigation can provide an efficient network of inland transportation. An efficient transport infrastructure provides mobility, flexibility and cost-effectiveness. Now-a-days, Government has recognized the need of actively promoting the IWT sector for it to take a reasonable share in the inter-modal mix of inland transport.

1.4	Fuel Efficiency

Water transport, makes the least demand on energy resources. It is an established fact that it requires less power to move an equivalent tonnage on waterway. One Horse Power is known to move 150 kg of cargo on road, 500 kg on rail and 4000 kg on water. Hence the cost per tonne kilometre is the lowest in the case of inland water transport is shown in Figure 1.1.

Total kilometres 1ton can be carried per litre of fuel :(1 gallon = 3.78litres)

Source:“Environmental Advantages of Inland Barge Transportation”,U.S. Department of Transportation, Maritime Administration, August, 1994.

By road: 25km

By rail   : 86km

By IWT : 219km

Figure 1.1.Relative Energy Efficiency

1.5	Emissions

Pollutants (in pounds) produced in moving one ton of cargo 1,000 miles

(1pound = 453grams). Table 1.1 shows the impacts of air pollution w.r.t IWT.

Table 1.1: Impact of Air Pollution caused by IWT

| IMPACT OF IWT ON AIR POLLUTION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Air pollution for transport of one ton/km of cargo (in grams) |  |  |  |  |
| Pollutants | road | water | National level Total inland cargo carried = 1200 Billion ton/km Every 1% of cargo diverted to IWT will reduce CO2 emission by 17,800 ton CO emission by 6,700 ton |  |
| Carbon Monoxide | 0.92 | 0.25 |  |  |
| Sulphurdioxide | 0.32 | 0.06 |  |  |
| Nitrogen oxide | 2.84 | 0.27 |  |  |
| Carbon dioxide |  |  |  |  |

Inland waterways can relieve the burden on port infrastructure - especially in the maritime ports - and on the surrounding rail and road infrastructure, thereby reducing congestion, energy and fuel costs. In terms of energy consumption per tonne of freight, inland shipping is already regarded as one of the most efficient means of transport.

(1 BARGE)

(15 RAIL CARS)

(1 BARGE)

(60 TRUCKS)

Figure 1.2: Equivalent Capacity of Transport Mode

1.6	Background of Study

Thiruvananthapuram  district accounts for about 5.6% of the total area of the state and comprises all the three natural regions viz: coastal area (543.35 sq.km), the midlands (874.54 sq.kms) which comprise mostly the Attingal, Neyyattinkara uplands and hilly regions (768 sq.kms) comprising Ponmudi, Agasthiar forest hills. It has three major rivers, besides a number of small lakes and backwaters that run along its 75 kms long coast.

Thiruvananthapuram, the capital of State has a network of canals, backwaters and river system which have played a predominant role in the maritime history of the region.  These waterways were the main highways of trade and commerce in the yesteryears and which was the lifeline of the State's economy is dysfunctional due to neglect and ignorance. The water transport helped to connect the islands and remote villages with the main land. It can be seen from history of Kerala that Inland waterways length of 1895 Km also have contributed to the unification of the state through an uninterrupted communication from North of Kerala to the South. The maritime importance of various ports like Thiruvananthapuram, Kochi , Kollam, Allappuzha, Beypore etc was mainly due to the presence of waterways extending almost to the midlands.  Hardly 20% of the waterways are in use for navigation and the remaining sections are being used as drainage channel for the city’s waste water and effluents from various land uses. The management of important canals and their surroundings in the City are vital for sustainable development of the region.

Trivandrum-Shoranur Canal popularly known as T S Canal was commissioned in stages under the three erstwhile States of Madras, Cochin and Travancore between 18th and 19th century. The portion between Channankara to Trivandrum was first done to effect through-traffic from Varkala to Trivandrum. Then, Paravoor canal was constructed to link backwaters of Paravoor and Edava. Quilon canal came next connecting Paravoor and Asthamudi lakes. Thus, the waterway route in this region was completed by 1880 to have through movement of goods and passenger-traffic between southern parts of Travancore to central parts of the State.

High proximity to the major tourist spots like Kovalam beach, Veli estuary, Veli tourist village, Akkulam boat club reveals the feasibility of the project. In addition to this there are several tourist attractions which are unique and special to Trivandrum only and also Trivandrum International airport, Central railway station and Central bus stand are easily accessible from the project area.

Thiruvananthapuram city has inherent problems of traffic congestion, heavily populated central core, intense development of land uses and inadequate mass transport system. The growing demand for transportation in the city cannot be met with the existing infrastructure facilities alone. There has to be greater emphasis on developing additional transport facilities with matching infrastructure development to meet the future travel needs of the people.

Development of waterways has several advantages namely sustainable means of transport, better mechanism for flood control, preservation of ecosystem, enhancing the recreational activities and tourism potential of the region, besides decongestion of heavily trafficked corridors in the city.

Improving Kovalam - Akkulam stretch of canal will facilitate economising the transportation cost of the passenger and cargo movements to airport in addition to the tourism potential of the Thiruvananthapuram city.

Realizing the need to develop the waterway with utmost priority, Inland Navigation Directorate has entrusted NATPAC to undertake the study for improvement of Kovalam - Akkulam Stretch of the T.S canal in Thiruvananthapuram District for Inland Navigation, Tourism and Recreational Purpose. This final report is prepared towards this end.

CHAPTER II

SCOPE AND OBJECTIVES OF THE STUDY

The scope of the study is to chart out proposals for the revival of T S Canal for Inland Navigation and Tourism Promotion in Thiruvananthapuram Region to ensure sustainable transport. It includes the stretch between Kovalam and Akkulam of T.S canal for a length of 18.49 km. Towards realizing the broad objective, the specific tasks of the study are given below;

- Assess the present status of the canal between Kovalam and Akkulam;

- Conduct topographic/hydrography survey using Differential Global Positioning System (DGPS), Electronic Total Station(ETS) and Echo sounder equipments to estimate the dredging quantity to maintain uniform width of 14 metre and draft of 1.7 metre;

- Assess the present condition of canal/road cross structures, and suggest appropriate improvements;

- Propose facilities to the canal like bank protection, navigational aids;

- Propose suitable sewage pipelines of houses and other polluting sources on the banks to Valiathura (Muttathara) sewage treatment plant;

- Fix up the canal boundary by referring to survey/resurvey sketches.

Improvement proposals have been formulated with tourism, transportation, economy and environment as the main criteria.

CHAPTER III

STUDY AREA

3.1	Geography and Climate

Thiruvananthapuram is situated in 8º 25' North latitude and 76º 55' East longitude. The city is well connected by road, rail and air transport. A city of hills and valleys, the ground level varies from 0 to 76 m above Mean Sea Level. Geologically the city is characterized by sandy soil along the west and red laterite soil along the east. Granite deposits exist in some parts of the city especially at Peroorkada and Thirumala. A fairly uniform temperature characterizes the climate of Thiruvananthapuram with high humidity throughout the year due to the city’s proximity to the sea. The average annual rainfall in Thiruvananthapuram is 1,803 mm, which is lower than other parts of State. On average there are 102 rainy days in a year. The hottest months are from March through May with an average maximum temperature of 31.7º C. The average daily minimum temperature of 22.5ºC occurs during December to January.

3.2	Population

Census 2011 had put Thiruvananthapuram district's population at 33,07,284 has a high density of population which, is 1509 persons per sq.km, against 1476 in Census 2001.  The sex ratio in the district is 1088 females per 1000 males. Urbanization and urban population growth are pointers towards the change in the occupational pattern of the community, from agriculture and allied livelihoods to industrial and other non-agriculture occupations.

Thiruvananthapuram city alone accounts for 68% of the urban population in the District. Thiruvananthapuram Development Authority (TRIDA) area is 296 sq. km encompassing 142 sq. km. Municipal Corporation area and 154 sq. km of adjoining ten grama Panchayats. Accordingly, the average population density in the TRIDA area was 3,800 persons per sq. km. In the city’s central area and the along the coastal belt, the population density is as high as 5,300 persons per sq. km with only 2,500 persons per sq. km in the outer fringes of the city.

3.3	Economy of Thiruvananthapuram

The economy of Thiruvananthapuram mostly consists of tourism, information technology, rubber plantation, coffee production, tea production and education. There are many manufacturers such as Travancore Titanium Products Ltd and English Indian Clays.

- 3.4	Manufacturing Industries

In Thiruvananthapuram there are many medium and large scale industries which fall under central sector, state sector, co-operative sector, joint sector and private sectors. The Kerala State Industrial Development Corporation (KSIDC) is a public sector establishment to set up small scale and large scale industries effectively. The registered working factories in the city include oil mills, cashew factories, cotton textiles, saw mills, printing units, rubber industrial units, chemical units, match factories, general engineering units and automobile workshops. The S.M.S.M. Institute in Thiruvananthapuram is a major institution through which the products of the handicraft industries are marketed.

There is an industrial estate at Pappanamcode and an industrial development centre at Kochuveli. Traditional industries such as coir and handloom are now faced with several problems. Handloom weaving is prevalentat Balaramapuram, Amaravila, Kulathoor and Chirayinkeezhu, which are located in the suburbs of the city. It is estimated that clothes worth Rs.950 lakhs are annually produced in the handloom sector. Keltron (Kerala State Electronics Development Corporation) has made inroads into electronic markets throughout the country. The Kerala State Sericulture Co-operative Federation Limited (SERIFED) had declared Balaramapuram as a "Silk Village" in 2004 and the silk yarn produced at different centres in Kerala is processed to Fabric at Balaramapuram.

Companies such as Travancore Titanium Products Ltd (TTP), the leading manufacturer of anatase grade titanium dioxide in Asia, English Indian Clays Limited, Kerala Automobiles Ltd, the leading public sector company in manufacturing three-wheelers in south India are located in the city. Hindustan Latex Limited is an Indian giant in the manufacturing of latex-based products, including condoms. They currently hold a license to manufacture Ormeloxifene, a form of birth control. HLL is headquartered in the city. BrahMos Aerospace Private Limited, the manufacturer of cruise missiles has the manufacturing unit in the city.

3.5	Tourism

Tourism has also contributed heavily to the economy of Thiruvananthapuram. Thiruvananthapuram, is a destination for chartered flights to India for Medical tourism.

3.6	Information Technology

Technopark was established for the development of electronics and information technology in the State. It is India's first industrial park dedicated to electronics, software, and IT ventures. Started 1995, the campus at Thiruvananthapuram city covers an area the 330 acres (1,335,462.6 m2) campus with 4,000,000 sq ft (370,000 m2). of built-up space available currently and another 2,000,000 more sqft of built up space coming up, is now home to over 260 companies. Over 35,000 IT professionals are working here. The companies include one CMMI level 5 and PCMM level 5 company, four CMM Level 5, two CMM Level 3 and several ISO 9001 certified companies. Technopark is undergoing major expansion with the development of the Technocity project.

Thiruvananthapuram was rated as the best 2nd tier metro with IT/ITES infrastructure, and second in terms of availability of human talent. The district contributes 80% of software exports from the state. Technopark also houses global majors like Oracle Corporation, Infosys, TCS, HCL, Visual Graphics Computing Services, Ernst & Young Global Shared Services Center, Allianz Cornhill, UST Global, Tata Elxsi, IBS Software Services, NeST Software, SunTec Business Solutions etc.

3.7	Aerospace

The Indian Space Programme came into being on 21 November 1963 with the launching of a sounding rocket from Thumba in Thiruvananthapuram city borders. This Thumba Equatorial Rocket Launching Station (TERLS) ultimately blossomed into various centres of the Indian Space Research Organisation (ISRO). Vikram Sarabhai Space Centre (VSSC) is the prime centre engaged in design and development of launch vehicles. The ISRO under the Department of Space, Government of India has the VSSC at Thumba and Liquid Propulsion Systems Centre (LPSC) at Valiyamala Nedumangadu in Thiruvananthapuram district. The ISRO Inertial System Unit (IISU) of the ISRO and the Reinforced Plastic Centre (REPLACE) function from Vattiyoorkavu in the city.

3.8	Study Area

The study area stretches from Kovalam to Akkulam canal of the Parvathy Puthanaar is about 18.49 km runs almost parallel to Sea and NH bye pass road. Though the waterway is interspersed with several natural and artificial impediments such as over bridges, locks, etc. this can be made navigable by adopting suitable time bound implementation programme. The waterway is passing through Panathura, Edayar island, Moonattumukku,  Poonthura, Vallakadavu, Trivandrum airport, Vempalavattom and ends at Akkulam lake. The study area map is shown in Figure 3.1.

There are a large number of beach resorts in and around Kovalam. The sea port of Vizhnjam is about 3 km away and famous for its special varieties of fish, old Hindu temples, big churches and a mosque. The Proposed International Transhipment Terminal at Vizhinjam is also closer to Kovalam.

Kovalam was among the most prominent tourist spots in India during the hippy era. It still has a high status among tourists, who arrive mostly from Europe and Israel. Kovalam is finding a new significance in the light of several Ayurvedic salons, and recuperation and regeneration resorts which provide a wide variety of Ayurvedic treatments for tourists.

Figure 3.1: Study Area Map of Kovalam - Akkulam Canal

Akkulam is a neighbourhood of Thiruvananthapuram city. It is about 10 km from Thiruvananthapuram city centre. It is a well known for the picnic and backwater destination Akkulam Lake, an extension of Veli Lake, where it joins with the sea.

Poonthura is a breath-takingly beautiful place. Karamana River falls into the Arabian Sea at Poonthura Pozhi.

Edayar Island is an unspoiled land in Thiruvananthapuram District of Kerala. It is in the truest sense a getaway for holiday makers, peace seekers and tired nerves. Edayar lies near Thiruvallom, about 7 km from Thiruvananthapuram Central Railway Station, on the way to the international beach resort of Kovalam. The island is spread over 2 km in length and half-a-km in width. The temple, kavu and church are main attractions. An ancient Parasurama temple, on the banks of the Karamana River, is nearby. Edayar Island can be reached by ferries. Alternatively one can also opt for a narrow iron bridge near Thiruvallom to walk into the island. Trivandrum International Airport is the nearest airport.

CHAPTER IV

METHODOLOGY

4.1	Literature Review

All the previous studies conducted by various agencies are reviewed in terms of estimation of dredging quantity, type of bank protection, operation of vessel, socio-economic profile, population and its forecasting, water demand, sanitation, public health, sewage and sewerage disposal techniques, development proposal, cost estimates, investment details etc.

4.2	Collection of Secondary Data

Secondary data was collected to know about the previous condition of the canal stretch, utility etc. Discussions were held with engineers of Inland Navigation Department, Irrigation Department, Inland Water Authority of India and District Tourism Promotion Councils.

4.3	Reconnaissance, Inventory and Condition Survey

A detailed reconnaissance was carried out to assess the present condition of the canal, details of cross structures like road bridges, rail bridges, foot bridges, pipelines, electric lines, telephone lines etc, existing bank protection, road connectivity, land use and plan the course of action.

4.4	Socio-economic Survey

This was done through a detailed questionnaire survey which provides details of demography, type of buildings, built up area, drainage facilities, sanitary facilities, livestock details, sources of water, availability of water, water demand, purpose of water, disposal of waste water and solid waste and usage of canal. A sample questionnaire is given in Annexure I.

4.5	Population, Water Demand, Waste water and Solid Waste Generation

From the socio economic survey, the existing population along the study stretch was obtained. The population projection is determined by various methods like incremental increase method, graphical slope method and arithmetic mean method. The design period for the population projection is taken as 2025. The water demand, generation of waste water and solid waste are also calculated for the projected population.

4.6	Sewer line

Sewer is an artificial conduit or system 	of conduits used to remove sewage and to provide drainage. Sewage is the mainly liquid waste containing some solids produced by humans which typically consists of washing water, faeces, urine, laundry waste and other materials from household.

Designing of sanitary sewer involves estimation of waste flow rates for the design data and evaluation of local condition, which may affect the hydraulic operation of the system; the selection of the hydraulic –design equation, maximum and minimum sizes; maximum and minimum velocities and pipeline alignments.

4.7	Environmental Status and Issues

Water samples were collected from locations where there is change in the characteristics and tested in the laboratory for parameters like temperature, pH, conductivity, dissolved oxygen, BOD, Total suspended solids, Total dissolved solids, chloride, turbidity, total hardness, calcium, magnesium, iron, sulphate, nitrite, nitrate, inorganic phosphate, total nitrogen and total phosphorous. Sediment samples were collected using grab sampler and tested in the laboratory for parameters oxygen demand, settling velocity, organic carbon, phosphorus, iron and sulphide.

4.8	Topographic Survey

The topography survey was conducted along Kovalam - Akkualm stretch and its adjacent area up to 50m on both side from the centreline of the canal for preparing base map of the study area using advanced survey equipments like Differential Global Positioning System (DGPS) and Total Station. The Primary Reference Point (PRP) at Sastra Bhavan was established with respect to International Geodetic Station (IGS) of IISc Bangalore. The Secondary Reference Points (SRP's) i.e 10 GPS Bench Marks along the canal was established with respect to PRP using Leica 1200 Series DGPS/GS 14 and CS10 in static mode with milli meter accuracy. The observed raw data was post-processed with Leica Geo Office (LGO) software and the WGS84 coordinates(Latitude, Longitude, Ellipsoidal height) are converted into UTM coordinate system i.e the local coordinate system (Easting, Northing, Height). The EGM96 Model was used in the study for geiod separation.

The observed ellipsoidal height was converted into orthometric height i.e with respect to Mean Sea Level (MSL). The bench mark of War Memorial Pillar, Palayam, Thiruvananthapuram was chosen for level conversion. The height of War Memorial Pillar was transferred to Chackai Railway Bridge bench mark using auto level instrument and the same was transferred to all the bench marks established by NATPAC along the study stretch. The Total Station (PENTAX) which can measure angles with an accuracy of about 2 seconds of arc and distances up to 500 meter with an accuracy of 5mm + 5ppm was deployed for detailing survey. The open traverse survey was conducted between the Secondary Reference Points. It was started at a GPS Reference Station and be closed at the other Reference station. The accuracy of the closure angles was determined mathematically and the same was distributed to all the stations.

The detailing of existing features such as canal, road, culvert, bridge, railway line, building, compound wall, fence, water body, sea wall, drainage, retaining wall, electric post, telecommunication box, major bunds etc were measured and the boundary of the canal is  demarcated. It was done by measuring the distances and angles to a special optical target; it allows to compute the plan positions and heights of points. The levelling of the study area was carried out in an equal interval.

The measured raw data was post-processed with AUTO PLOTTER software. The obtained dxf.file was transferred into dwg. file. Plotting of all the features has been done using AUTOCAD software and the final layout of the existing canal and its adjacent area is prepared in 1:1000 scale. To fix up the boundary of canal in the layout by referring to survey/resurvey sketches of Revenue Department was also be done.

4.9	Hydrographic Survey

This survey has been conducted to get the canal width, depth etc. using ultra sound technique (Doppler Principle) and also to find the velocity of running water.

Hydrographic survey was conducted using echo sounder for the canal portion wherever possible i.e. from Kovalam to Akkulam. The existing bed profile of the canal and water level were collected at 25 m cross sectional interval using a single frequency echo sounder equipment. The estimation of dredging quantity was done using the hydrographic survey data.

4.10	Improvement Proposal

Development proposals were identified to make the canal navigational which includes desilting, widening, protection of banks, improvement of the canal, provision of navigational aids, improvement of approach roads, reconstruction of cross-structures with inadequate clearance, sewage and sewerage treatment facilities, canal protection and safety measures etc.

In order to sustain the utility of the canal, locations were identified to develop spots for tourism activities. Canal beautification schemes are also outlined

4.11	Cost Estimation

The activities required for development of the canal for navigation was identified and the quantities were computed. The rates are arrived in consultation with the user department to estimate the project implementation cost.

CHAPTER V

PRESENT CONDITION OF THE WATERWAY

5.1	General Characteristics

The width of the canal, varies from 10-500m and depth ranges between 0.1 and 5m. At Thiruvallom, Karamana River meets this waterway. This can serve as a feeder waterway through which the cargo can be transported from the influence area of the Karamana River. After Munnattumukku Bridge near Thiruvallom, the waterway divides in to two arms, which then passes around Edayar Island and meets again at Poonthurapozhi. The left branch of the waterway which passes through Thiruvallom and meets with Karamana River is taken as the route for the waterway. Amayizhanchan Thodu, Thekkenkara canal, KariyilThodu and Karamana River are major feeders of the canal. The canal is completely filled with sand near the Panathura Subramanya Swami Temple about 200m.

Most of the canal in this reach passes through level ground and the height of cutting required for widening will be limited within 3m above the required water level. At present, the canal stretch is completely polluted due to human intervention and there is, in effect no visible flow of water. The banks are not protected for about 90%.But while widening the canal the banks has to be protected by stone pitching where height of cutting is above 2m.The soil is sandy and a side slope of 1:2 is to be provided. The major structure to be reconstructed is the SM Lock which is completely abandoned as it is very old.

At present T.S Canal from Moonattumukku to Akkulam Kayal is not fully navigable as the canal is narrow, highly silted, blocked by weed growth, cross structures with less clearances etc. The canal is highly polluted due to dumping of domestic waste as well as waste from sewage treatment plant, making the water unsuitable for growth of aquatic life. Untreated liquid waste from houses and hotels are dumped into the canal, which has boosted the breeding rate of mosquitoes and out breaking of waterborne diseases to the dwellers on both sides of canal. Good service road parallel to the canal exists. There are 10 road bridges, 2 rail bridges, 6 foot bridges, 1 aqueduct and a lock exits at this section. World vegetable market, Kochuveli Railway Station, Trivandrum International Airport, Sewage farm, All India Radio are situated near the canal.

The section wise details are given as below:

5.2	Kovalam - Panathura

Kovalam is the starting point of the study area and the total length of this stretch is 1.85km. It has both navigable and non-navigable portion. This stretch almost parallel to the Arabian sea and tourist attraction is predominant. Taj and Samudra hotel are situated on the banks of canal. At Panathura, near Subramania Swamy Temple the canal is filled about 200m and is a major bottleneck for navigation. A paved road is constructed in this filled area and two houses are existing. This portion has to be dredged and a bridge is proposed to connect with NH bye pass road. Coir retting activity is prevailing in this stretch, so that, the navigable portion is hindered. No cross structures are available in this stretch. The road connectivity up to 500m is available in the width of 2.75m on eastern side of the canal. Figure 5.1,5.2,5.3 and 5.4 shows the status of the canal.

Figure 5.1: Starting Point of Kovalam - Akkulam Canal

Figure 5.2: View of Canal at Panathura

Figure 5.3: Filled Portion of the Canal at Panathura

Figure 5.4: Another view of Filled Portion of the Canal at Panathura

5.3	Panathura - Edayar Island South

This stretch starts from the filled portion to the starting point of Edayar island or southern side of the island. The length of the stretch is 1.86km. The deposition of coconut husks in the canal is blocking the flow of water. At Edayarisland the canal is connected with Sea having width of 400m. It serves the canal's self purifying capacity but it is seasonal. The sand bar formation at the mouth of Sea has also to be dredged and the movement of sea water into the canal has to be restored. Deprived road connectivity to the canal is to be improved. The existing bank protection is to be strengthened. This stretch do not have any cross structures. Figure 5.5 and 5.6 shows the view of the canal.

Figure 5.5: View of Sea opening at Edayar Island South

Figure 5.6: View of private boat operation at Edayar Island South

5.4	Edayar Island South - Thiruvallom

The length of this stretch is 1.9km. The canal is divided into two channels, one is passing parallel to sea and connects at Moonattumukku and other one is running parallel to NH bye pass. Through Thiruvallom it joins at Moonattumukku. This stretch is having sufficient width and depth with fewer dredging is required and is navigable up to Thiruvallom. Karamana river is joins at Thiruvallom. This stretch is not consisting any cross structures and the approach road is only the NH bye pass. The banks are protected almost throughout the stretch and the up gradation is needed to prevent from further degradation.

5.5	Edayar Island South- Moonattumukku

The length of this stretch is 2.25km and it is running parallel to sea. No cross structures are existing in this stretch. The east bank of the canal has to be protected unless the due to tidal action the erosion will be taken place on Edayarisland. At Moonattumukku, the deposition of solid waste in the canal blocks the flow of water. Figure 5.7 shows the view of canal from Moonattumukkuroad bridge.

Figure 5.7: View of canal from Moonattumukku Bridge

5.6	Thiruvallom - Moonattumukku

This section is having length of 580m. This stretch is not navigable due to siliting, weed growth and heavy pollution. Two cross structures are existing in this section namely steel bridge and foot over bridge. Both are structurally weak in its condition and are to dismantled and reconstructed. The approach is connecting from Thiruvallom to this canal and is good in condition. No bank protection is existing and it is also to be constructed. Figure 5.8 and 5.9 shows the view of bridge and canal.

Figure 5.8: View of Steel bridge connects Edayar Island

Figure 5.9: View of canal from Steel bridge

5.7	Moonattumukku - Ponnara Bridge

The length of the stretch is 3.34km. This section is not navigable due to narrow width and enormous amount of solid waste deposition at various sites. Nearly 350 families are situated on the banks of the canal and they do not have any healthy sanitation facilities. So the section of canal is serving as drainage and dumping of wastes. It is essential to provide sanitation facilities like sewage pipeline for liquid waste and solid waste management system to the local people and the canal has to be prevented. The Common Effluent Treatment Plant (CETP) is situated at Muttathara is closer to canal. This being used for connect the sewage pipeline along the canal.

Four cross structures at Moonattumukku, SM lock, Steel bridge near Muttathara and Aqueduct at Muttathara are existing in this section. The SM lock is not functioning and the condition of the structure is also feeble. Other structures better in condition but to be renovated. The road connectivity with canal is good and is well connected with Thiruvallom, Beema Pally and Muttathara. The banks are in damaged condition and even some of the places it is all collapsed. Figure 5.10,5.11,5.12,5.13,5.14 and 5.15 shows the view of canal.

Figure 5.10: View of Canal from Moonattumukku Bridge

Figure 5.11: View of SM Lock

Figure 5.12: View of Solid waste dumping at SM Lock

Figure 5.13: View of liquid waste discharge near SM Lock

Figure 5.14: View of Steel Bridge near Muttathara

Figure 5.15: View of Aqueduct at Muttathara

5.8	Ponnara Bridge - Chakka Railway Bridge

The length of this section is 3.52km. Trivandrum Air port, Chakka Railway bridge and NH bye pass are well connected with this canal. Currently this section is not navigable due to siltation, weed growth, environmental issues and poor vertical and horizontal clearances of existing cross structures. Six cross structures are presented in this section. Two road bridges are connecting with Domestic and International airports situated at Ponnara and near Ananthapuri hospital respectively. In addition to this, two road bridges at Vallakadavu and Puthenpalam are existing in this section. The foot over bridge near Kairalinagar is also presented in this section is not structurally in good condition. The Puthenpalam and Kairali foot bridge has to be dismantled and reconstructed. This section is also facing serious environmental threats by the settlers on the banks through discharging the waste and dumping the solid matters into the canal. The bank protection has also to be strengthened in this section. Figure 5.16,5.17,5.18,5.19 and 5.20 represents the view of canal and its condition.

Figure 5.16: View of Canal from Ponnara Bridge

Figure 5.17: View of Canal from Ponnara Bridge

Figure 5.18: Solid waste dumping near Ponnara Bridge

Figure 5.19: View of PuthenPalam Bridge

Figure 5.20: Discharging outlets from Settlers

5.9	Chakka Railway Bridge - Vempalavattom Bridge

The length of stretch is 2.32km. This section consists of two road bridges at Chakka, two rail bridges at Chakka, two foot over bridges between Chakka and Karikkakom, one steel bridge at Karikkakom and one foot over bridge near Vempalavattom. The foot over bridges are very damaged condition and these all are has to be dismantled. The canal section is narrow and its needed minimal amount of dredging for navigation. The banks of the canal have to be protected. The sewage pipeline and other sanitation facilities have to be provided to the settlers on the banks to avoid the environmental issues into the canal. The canal is well connected with roads on both the sides. Figure 5.21 and 5.22 shows the view of canal in this section.

Figure 5.21: View of Chakka Railway Bridge

Figure 5.22: View of Foot Bridge NearKarikkakom

5.10	Vempalavattom Bridge - Akkulam Boat Club

This section is having length of 3.12km. The canal is having sufficient width and depth. Due to weeds growth, the canal is not navigable at present and after cleaning, the vessel operation may be taken place. This section consists of two road bridges, one at Vempalavattom and other one is situated Akkulam bye pass road of NH bye pass. The world vegetable market and Kochuveli railway station, Veli and Akkulam tourist villages are the known landmarks which are situated closer to the canal. The canal is bifurcated at 200 m away from Vempalavattom bridge and it connects Veli tourist village. The settlers on this section is nil and the environmental issues are relatively less. The banks of the canal has to protected. The canal ends at Akkulam boat club, due to heavy weeds growth nearer to club is causing hard to move with vessel.  Figure 5.23 and 5.24 shows the existing condition of the canal.

Figure 5.23: View of Bifurcation of Canal to Veli and Akkulam

Figure 5.24: View of Akkulam Canal End

CHAPTER VI

RESULTS AND DISCUSSION

6.1	Canal Development

In order to develop the canals to a model waterway for sustained use as a means of tourist destination and for cargo and passenger movement, certain improvement proposals which are to be done immediately are charted out.

6.2	Canal Development Standards

The standard for development of the waterway depends on the type of traffic envisaged in the section. This should meet the following agenda.

- Should cater to the travel demand of passengers and cargo though the waterway.

- Promote water based recreation facilities thus facilitating enhanced tourism in the region.

- Ensure safe and sustainable navigation though the canal by negotiating the existing physiographic constraints.

The classification of vessels operated in inland water canal is as given in Table 6.1. The specification for the vessels are given in Table 6.2.

Table 6.1: Classification of Vessels

| Sl. No. | Traffic Type | Vessel Type |
| --- | --- | --- |
| 1. | Local passenger | - i) Local crafts ii) FRP utility boats |
| 2. | Cargo | - i) Traditional cargo vessels (KettuValloms) - ii) Self propelled Barges |
| 3. | Tourists | - i) FRP pleasure boats - ii) Speed boats - iii) House boats (Traditional) - iv) Canos and kayaks |

Table 6.2: Specifications for Vessels

| Sl No. | Type | Length (m) |  | Max Width (m) |  | Depth (m) | Draft (m) | Propulsion | Capacity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I. PASSENGER VESSELS |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Traditional (vallom) | 2 - 10 |  | 2.1 |  | -- | <1 | rowing |  |  | -- |
| 2 | Utility passenger boats(FRP and Plywood) | i)2.1 ii)7.0 |  | 1.53 1.3 |  | 0.8 0.8 | -- -- | Manual rowing Rowing |  |  | -- -- |
| 3 | Utility passenger boats (FRP and Plywood) | i)3.6 ii)4.84 iii)5.45 iv)6.0 |  | 1.5 1.82 1.87 1.9 |  | 0.43 0.71 0.73 0.76 | -- -- -- -- | 7.5to40 HP 7.5to40 HP 7.5to40 HP 7.5to60 HP |  |  | 250 kg 600 kg 700 kg 800 kg |
| II. CARGO VESSELS |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Traditional | 4-12 |  | 3 |  | -- | 1.3 | Rowing |  | 4 to 8 T |  |
| 2 | Speed Barges | 12.2– 30 |  | 4.25 – 8 |  | -- | 1.5 (0.6 for barge 1.2 for propeller) | 300 HP |  | 100T– 350 T |  |
| III. TOURIST VESSELS |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Speed boat |  | i)5.66 ii)6.3 | 2.43 2.43 |  | -- -- | 0.36 0.36 | 100HP to 200 HP upto 250 HP |  | 9 to 10 persons 11 to 12 persons |  |
| 2 | House boat |  | 20 | 3.40 |  | -- | 1.2 | OBM |  | -- |  |
| 3 Canos and Kayaks |  |  |  |  |  |  |  |  |  |  |  |
| i) ii) iii) iv) vi) vii) viii) |  |  | 6.6 | 0.9 |  | 0.4 | -- | 0.5 to 4 HP |  | 300 kg |  |
|  |  |  | 9.4 | 1.4 |  | 0.7 | -- | 7 to 25 HP |  | 1500 kg |  |
|  |  |  | 10.6 | 1.5 |  | 0.68 | -- | 7 to 40 HP |  | 2500 kg |  |
|  |  |  | 11.2 | 1.68 |  | 0.90 | -- | 12 to 40 HP |  | 2800 kg |  |
|  |  |  | 12.4 | 1.8 |  | 0.90 | -- | 12 to 40 HP |  | 3200 kg |  |
|  |  |  | 13.0 | 1.88 |  | 0.90 | -- | 15 to 40 HP |  | 3600 kg |  |
|  |  |  | 13.9 | 1.9 |  | 0.90 | -- | 15 to 60 HP |  | 4000 kg |  |
|  |  |  | 14.8 | 2.0 |  | 0.90 | -- | 15 to 60 HP |  | 4400 kg |  |

Source: NATPAC Survey, 2005

6.3	Hydrographic Survey

The Primary Reference Point at Sastra Bhavan, Pattom was established with respect to IISc Bangalore IGS station. The post-processed WGS84 coordinates of PRP is given below;

Latitude: 8˚31'18.27368"N

Longitude: 76˚56'29.84724"E

Ellipsoidal Height: -37.4496m

The obtained ellipsoidal height was converted into orthometric height with respect to Benchmarks of Survey of India and the height of the PRP is 57.6094m with respect to Mean Sea Level (MSL).

The Secondary Reference Points (SRP's) along the canal were established  with respect to PRP Sastra Bhavan and the UTM coordinates of the Reference stations are shown in Table 6.3.

Table 6.3: Geo Coordinates of GPS Reference Stations

| POINT ID | EASTING (m) | NORTHING (m) | HEIGHT (m) | REMARKS |
| --- | --- | --- | --- | --- |
| Kovalam | 716887.7851 | 929767.6301 | 3.125 | Northern end of Taj hotel which is closer to compound wall |
| Panathura Temple | 716230.3851 | 930879.7519 | 2.330 | In front of temple and is between sea wall and panchayat road |
| Moonattumukku | 714613.0521 | 933763.8836 | 1.840 | Under the road bridge on right bank of the canal |
| SM Lock | 714347.0835 | 934335.8618 | 6.710 | Eastern end of SM lock |
| Muttathara Steel Bridge | 713356.7749 | 935867.4928 | 3.225 | Under the foot over bridge on right bank of the canal |
| Vallakadavu | 712681.4696 | 937745.3243 | 2.940 | South end of culvert on right bank of the canal |
| Chackai Road Bridge | 711499.5858 | 939109.6757 | 2.150 | Under the road bridge on right bank of the canal |
| Chackai Railway Bridge | 708775.9149 | 942188.3085 | 2.310 | Under the railway bridge between two railway tracks |
| Vempalavattom | 709480.5981 | 941065.9172 | 1.790 | Under the Karikkakom road bridge on left bank of the canal |
| Akkualam | 708775.9054 | 942188.3068 | 1.850 | Boundary of walkway and doctors colony |

The locations of benchmarks were jointly identified by Officers from Directorate of Inland Navigation and NATPAC Officials and as per PWD specifications the benchmarks were constructed at 10 locations along the study stretch. The reduced levels of the benchmarks were transferred from the permanent benchmark of War Memorial Pillar, Palayam and the details of benchmark stones are shown in Figure 6.1.

With respect of each reference stations the topography survey was conducted using Electronic Total Station. The features like canal boundary, road connectivity, cross structures including road bridges, railway bridges, foot over bridges, aqueduct, etc and land use pattern were surveyed as a part of topography survey.

Kovalam

Panathura Temple

Moonattumukku

SM Lock

Muttathara Iron Bridge

Vallakadavu

Chackai Rly Bridge

Vempalavattom

Chackai Road Bridge

The echo sounder instrument was deployed to measure the depth of the canal where ever possible at every 25m interval and at most of the places the depth was measured manually due to not navigability. The base map of study area is given in Volume II of the report.

Akkulam

Akkulam

Figure 6.1 Benchmarks

6.4	Dredging

The study area comprises of narrow reach from Moonattumukku bridge to Vempalavattom bridge for distance of 9.19 km. From the specification of the vessels being operated in inland water canals, as given in Table 6.2 above, it is seen that the maximum width for passenger and tourist vessels is 3.40 m for house boats. In the case of cargo vessels, the 100T barge has a minimum width of 4.25 m. Considering the above aspects, it is proposed a bottom width of 14 m and water depth of 1.7 m with side slope of 1:2.5 in the artificial canal portion.

The canal stretch under study is to be desilted to maintain the flow and to achieve the minimum required depth of water for movement of vessels. Cross-sections were taken at 25 m interval, unless there is any appreciable change in cross-section or alignment is noted.

The dredging quantity (percentage) with respect to section wise is shown in Figure 6.2. It can be noted that there is more siltation in  Poonthura pozhi to Moonattumukku sector. This is where the canal encounters deep cut in its course and emphasis the need for slope protection and drainage facilities to prevent future silting. The estimated dredging quantity is given in Table 6.4.

Table 6.4: Desiltation of Canal - Section wise

| S.No | Section | Dredging Quantity (m3) |
| --- | --- | --- |
| 1 | Kovalam – Panathura Temple | 19569.47 |
| 2 | Panathura Temple – Poonthura Pozhi | 33011.80 |
| 3 | Poonthura Pozhi - Moonattumukku | 45918.96 |
| 4 | Moonattumukku – SM Lock | 13182.41 |
| 5 | SM Lock – Muttathara Iron Bridge | 40149.71 |
| 6 | Muttathara Iron Bridge - Ponnara bridge | 20769.92 |
| 7 | Ponnara bridge – Vallakadavu | 26878.63 |
| 8 | Vallakadavu - Chackai Road bridge | 29941.61 |
| 9 | Chackai Road bridge - Chackai Rly bridge | 7346.38 |
| 10 | Chackai Rly bridge – Vempalavattom | 29633.15 |
| 11 | Vempalavattom - Akkulam bridge | 2058.58 |
| 12 | Akkulam bridge – Akkulam boat club | 3442.48 |
| Total |  | 271903.10 |

The total amount of dredging is estimated as 271903.10 cum.  The dredging quantity is calculated for 711 cross sections of Kovalam- Akkulam canal. The detailed result includes reduced levels and depth of each cross section is given in Annexure 2. The cross sectional drawings are given in Annexure 3.

Figure 6.2: Percentage of Dredging Quantity Vs Section-wise

The dredged material has to be disposed in the appropriate sites. The location for disposal of dredged materials are given in Table 6.5.

Table 6.5: Dredged Material Disposal Sites – Kovalam- Akkulam Section

| Sl. No | Chainage | Location |  | Land use | Lat long |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.00-1.00 | Kovalam (Ch. 0.36 km) | Rightbank | Coconut plantation | 8°24'21.33"N 76°58'16.12"E |
| 2 | 1.00-2.00 | Panathura (Ch. 1.47 km) | Right bank | Coconut plantation | 8°24'53.95"N 76°58'0.74"E |
| 3 | 2.00-3.00 | Panathura (Ch. 2.24 km) | Left bank | Coconut plantation | 8°25'11.29"N 76°57'49.69"E |
| 4 | 3.00-4.00 | Poonthura (Ch. 3.49 km) | Right bank | Coconut plantation | 8°25'40.72"N 76°57'30.42"E |
| 5 | 4.00-5.00 | Edayar (Ch.4.40 km) | Left Bank | Coconut plantation | 8°26'4.34"N 76°57'20.21"E |
| 6 | 4.00-5.00 | Edayar (Ch. 4.83 km) | Right bank | Coconut plantation | 8°26'18.20"N 76°57'19.37"E |
| 7 | 5.00-6.00 | Edayar (Ch. 5.58 km) | Left bank | Coconut plantation | 8°26'29.40"N 76°56'59.38"E |
| 8 | 6.00-7.00 | Poonthura (Ch. 6.08 km) | Right bank | Coconut plantation | 8°26'44.31"N 76°56'54.23"E |
| 9 | 7.00-11.00 | Enchakkal (Ch. 10.65 km) | Left bank | Coconut plantation | 8°28'50.04"N 76°55'47.42"E |
| 10 | 11.00-12.00 | Near Airport (Ch. 11.35 km) | Left bank | Coconut plantation | 8°29'9.20"N 76°55'29.85"E |
| 11 | 13.00-14.00 | Chakka (Ch.13.35 km) | Right bank | Coconut plantation | 8°29'59.81"N 76°54'48.61"E |
| 14 | 16.00 | Akkulam (Ch. 16.00 km) | Right Bank | Coconut plantation | 8°30'55.39"N 76°53'47.50"E |
| 15 | 16.00-17.00 | Akkulam (Ch. 16.62 km) | Left Bank | Coconut plantation | 8°31'6.01"N 76°53'51.00"E |
| 16 | 17.00-18.00 | Akkulam (Ch. 17.47 km) | Left Bank | Coconut plantation | 8°31'11.56"N 76°54'7.88"E |

6.5	Bank Protection

The fast movement of a watercraft can cause a great deal of turbulence in the waterway leading to bank erosion. Engineering techniques have been recommended to mitigate the impact of turbulence. Bank protection works are to be taken up in order to protect land and property on canal sides from flooding. Bank erosion problem due to turbulence would be more severe in the narrow stretches than in wider stretches. Strong bank protection is the best mitigation measure, which is suggested in the narrow stretches.

In Kerala, traditional canal banks have been protected using coconut logs driven into the canal beds on the bank along with woven coconut eliminates natural river edge habitats and may also encourage boat mooring in inappropriate areas.

In addition to the traditional canal bank protection measures, many alternative methods of bank protection, which are acceptable visually and functionally, are in use.  Some of them are:

- Rip - Rap (large boulders)

- Concrete blocks or large pre-cast units.

- Metal sheet piling.

- Gabions (wire mesh cage with tightly packed stones).

- Concrete pile – slab method.

- Geotextiles combined with vegetative cover.

In this study, Random rubble masonary or stone boulders strengthened with coir grids type is suggested and the length of 25.98 kms of the canal has to be protected and the details are given in Table 6.6.

Table 6.6: Stretches identified for bank protection

| Sl.No | Chainage |  | Bank protection in kms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Left Bank |  | Right Bank |  |
| 1 | 0 | 1 | 0 | 0 |  |  |
| 2 | 1 | 2 | 0.67 | 0.67 |  |  |
| 3 | 2 | 3 | 1 | 1 |  |  |
| 4 | 3 | 4 | 0.2 | 0.2 |  |  |
| 5 | 4 | 5 | 0 | 0 |  |  |
| 6 | 5 | 6 | 0.77 | 0.77 |  |  |
| 7 | 6 | 7 | 1 | 1 |  |  |
| 8 | 7 | 8 | 1 | 1 |  |  |
| 9 | 8 | 9 | 1 | 1 |  |  |
| 10 | 9 | 10 | 1 | 1 |  |  |
| 11 | 10 | 11 | 1 | 1 |  |  |
| 12 | 11 | 12 | 1 | 1 |  |  |
| 13 | 12 | 13 | 1 | 1 |  |  |
| 14 | 13 | 14 | 1 | 1 |  |  |
| 15 | 14 | 15 | 1 | 1 |  |  |
| 16 | 15 | 16 | 1 | 1 |  |  |
| 17 | 16 | 17 | 0.35 | 0.35 |  |  |
| 18 | 17 | 17.82 | 0 | 0 |  |  |
| TOTAL |  |  | 12.99 |  | 12.99 |  |
| GRAND TOTAL |  |  |  |  |  | 25.98 |

The study stretch is under proposal of National Waterways 3 extension. When the proposal is approved, reconstruction of bank protection is necessitated. The random rubble masonary material may be reused for further construction of bank protective measures.

6.6	Cross Structures

Different types of cross structures are observed in the stretch Kovalam to Akkulam. The details of road bridges, rail bridges, foot bridges, steel bridges, cable lines, electric lines, pipe lines etc are collected. 11 road bridges, 2 railway bridges, 7 foot bridges, 1 lock and 1 aqueduct are existing in the study area. The particulars of the cross structures are given in Table 6.7.

Table 6.7:  Details of Cross Structures

| Sl.No | Name of Bridge | Type of Bridge | No. of Span | Horizontal Clearance (m) | Vertical Clearance (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Panathura wooden footbridge | Temporary Wooden Foot Bridge | 1 | 4.5 | 2 |
| 2 | Edayar Road bridge | Road bridge, steel | 3 | 7.5 | 5 |
| 3 | Edayar foot bridge | Foot bridge, RCC | 3 | 9 | 5 |
| 4 | Moonattumukku Bridge | Road bridge | 3 | 11 | 6.5 |
| 5 | SM Lock | Road bridge, old lock | 1 | 5 | 5 |
| 6 | Muttathara steel bridge | Foot bridge | 1 | 20 | 4.5 |
| 7 | Muttathara Pipeline bridge | Foot bridge (Aqueduct) | 1 | 11 | 4.5 |
| 8 | Ponnara bridge | Road bridge | 1 | 15 | 4.5 |
| 9 | Puthanpalam | Road bridge | 1 | 10 | 2.2 |
| 10 | Vallakadavu bridge | Road bridge | 1 | 7.5 | 2.8 |
| 11 | Kairali foot bridge | Foot bridge | 1 | 20 | 4.5 |
| 12 | New airport bridge | Road bridge | 10 | 32 | 8 |
| 13 | Chakka old bridge | Road bridge | 3 | 19 | 3.8 |
| 14 | Chakka new bridge | Road bridge | 3 | 18.6 | 4 |
| 15 | Chakka rail bridge I | Rail bridge | 3 | 11.3 | 3.7 |
| 16 | Chakkal rail bridge II | Rail bridge | 3 | 12.6 | 3.9 |
| 17 | KarikkakamVayanasala bridge | Foot bridge | 3 | 9 | 3.74 |
| 18 | Karikkakam foot bridge | Foot bridge | 3 | 7.2 | 3.8 |
| 19 | Karikkakam temple bridge | Road bridge, Steel | 3 | 9.7 | 1.5 |
| 20 | Vazhavila old foot bridge | Foot bridge | 3 | 7.2 | 3.8 |
| 21 | Vempalavattom bridge | Road bridge | 3 | 17.8 | 3.8 |
| 22 | Akkulam bye pass bridge | Road bridge | 12 | 32 | 7.5 |

The temporary wooden foot bridge at Panathura, Edayarfoot bridge, Karikkakam foot bridge and Vazhavila old foot bridge has to be dismantled. The SM lock at Poonthura has to be reconstructed. The purpose of lock is to control the flood through canal and it is not in working condition. The road bridges namely Edayarroad bridge, Ponnara bridge, Puthanpalam, Vallakadavu bridge and Karikkakam are structurally weak and these bridges has to be reconstructed. The construction of road bridge at Ponnara is under progress. The condition of aqueduct at Muttathara has to be renovated.

6.7	 Socio Economic Survey

Socio economic survey was conducted across the study area stretching from Kovalam to Akkulam of about 18 km. The objectives included in the survey are determination of population, determination of water demand, generation of solid and liquid waste and their disposal, calculation of dredging quantity etc. The type of survey conducted is Questionnaire Method. Around 700 houses were surveyed.

6.7.1 Akkulam boat house to Chackkai

In this region the canal is running parallel to road. The survey was conducted on both banks. Canal is mainly used for the disposal of kitchen wastes and sewage. Canal is completely covered with weeds and it is moderately polluted. Wastes from automobile service centre and shops etc are also deposited. This chainage consists of about 150 houses on either bank.

6.7.2	Chackkai to Vallakadavu

The survey was conducted on either bank. RCC type houses are seen on left bank and they have their own waste disposal facilities. Thatched type of houses are seen on the right bank and they dispose degradable (human, kitchen and animal) and non-degradable (plastic, utensils, cloth) wastes to the canal and it is highly polluted. The water is highly contaminated in this stretch. Foul smell, mosquito breeding, skin diseases etc are some of the impacts due to this polluted water.

6.7.3 Vallakadavu to Thiruvallom

In this stretch, survey was conducted on both the banks. The entire wastes from the houses are dumped to the canal. Wastes from poultry farm etc are also dumped. Thus the water is highly polluted and is black in colour. Muttathara sewage plant is located in this area. The population in this stretch is so high when compared with others sections.

6.7.4 Thiruvallom to Kovalam

This stretch  is a tourist area consisting of several resorts, hospitals, and temples. Edayarisland is situated in this stretch and scarcely populated. The canal is polluted due to the disposal of waste from resorts, hotels, shops, houses and due to coir retting activities.

6.8	 Population, Water Demand, Waste water and Solid waste generation

It is resulted that the existing population on both the banks are 2422. The population between Akkulam - Muttathara is 1910 and the stretch Muttathara - Kovalam is having population of 512. The population projection for the year 2025 is calculated based on the Kerala Population growth rate and Figure 6.3 represents the population projection of the study area.

Figure 6.3: Graph of Population Projection

From this graph, the population for the corresponding years is obtained. Using this data, the amount of solid waste, waste water and water demand is calculated. The water demand for the existing population is calculated as the water demand of per capita is assumed 135 lpcd. The solid waste of 0.5 kg/day is assumed for existing population. The water demand for projected population is assumed as 250 lpcd and for solid waste generation it is assumed as 0.75 kg/day. The waste water generation for both existing and projected population is assumed 75 percent of the water demand. The projected population, water demand, waste water and solid waste generation are given in Tables 6.8 & 6.9.

Table 6.8: Results of socio economic survey from Akkulam –Moonattumukku

| Year | Population | Water Demand (lpcd) | Waste water (lpcd) | Solid waste (kg/day) | WD annually | WW annually | SW annually |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2013 | 1910 | 257850 | 193388 | 955 | 94115250 | 70586438 | 348575 |
| 2015 | 1916 | 479000 | 359250 | 1437 | 174835000 | 131126250 | 524505 |
| 2020 | 1936 | 484000 | 363000 | 1452 | 176660000 | 132495000 | 529980 |
| 2021 | 1941 | 485250 | 363938 | 1455.75 | 177116250 | 132837188 | 531349 |
| 2025 | 1946 | 486500 | 364875 | 1459.5 | 177572500 | 133179375 | 532718 |
| 2030 | 1961 | 490250 | 367688 | 1470.75 | 178941250 | 134205938 | 536824 |

Table 6.9: Results of socio economic survey from Moonattumukku to Kovalam

| Year | Population | Water demand (lpcd) | Waste water (lpcd) | Solid waste (kg/day) | WD Annually | WW annually | SW annually |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2013 | 512 | 69120 | 51840 | 256 | 25228800 | 18921600 | 93440 |
| 2015 | 517 | 129250 | 96937.5 | 387.75 | 47176250 | 35382188 | 141528.75 |
| 2020 | 523 | 130750 | 98062.5 | 392.25 | 47723750 | 35792813 | 143171.25 |
| 2021 | 525 | 131250 | 98437.5 | 393.75 | 47906250 | 35929688 | 143718.75 |
| 2025 | 527 | 131750 | 98812.5 | 395.25 | 48088750 | 36066563 | 144266.25 |
| 2030 | 532 | 133000 | 99750 | 399 | 48545000 | 36408750 | 145635 |

It is estimated that the projected population in 2025 will be 2473 in the study area. The expected annual water demand will be 2257 lakhs litre of water and the wastewater generation by the people will be 1692 lakhs litre. The sewage pipeline has to designed for this projected wastewater generation. The solid waste generation will be around 6.8 lakhs kg annually.

## 6.9	Waste Management

## 6.9.1 Sewage

The sewage from the settlers at the canal bank is inclusive of the discharges from latrines, urinals and stakes. The discharges from the bathrooms and the storm water (i.e. rain water) are also included in sewage. This is causing unhealthy conditions by producing bad smell, fly nuisance, bacteria, etc. The sources of water  is polluted which may cause water-borne incomes.  This condition exists along Moonattumukku - Vempalavattom stretch where the encroachers thickly populated.

## 6.9.2 Management of Sewage

Sewage is a major issue for environment management. Typically, the problem with sewage is that it causes massive nutrient loading in the canal ecosystem. Nutrient loading triggers algal blooms in the water leading to the loss of dissolved oxygen. The enrichment of water by nutrients, especially compounds of nitrogen and phosphorus causes an accelerated growth of algae and higher forms of plant life to produce an undesirable disturbance to the balance of organism and the quality of the canal water.

To avoid the insanitary issues of canal, following factors to be considered.

- Construct the community toilets with latest technologies as required on both sides of the canal;

- Provide drainage/sewerage facilities to the settlers;

- Conduct public awareness programmes to the local community;

- Involvement of NGO’s in collaboration with TRIDA or any other government agency to guard the canal from polluting by residents.

6.10 Design of Sanitary Sewer

The design of sanitary sewer pipeline is divided into two sections viz Kovalam to Muttathara Sewage Treatment Plant (STP) and Akkulam to  Muttathara STP. The design area of sewer pipeline is estimated as 4467 Ha and 80 % of activity is assumed as residential and remaining 20% of the area is considered as commercial. In an average of 3 dwelling units with 5 residents are assumed for determining the average daily flow (ADF). The estimated ADF for residential is 16081.2 m3/day and for commercial 160812 m3/day. The total ADF for the study area is 176893.2 m3/day. The estimated peaking factor is 2.0 and the Peak Design Flow is 393989.4 m3/day or 4.56 m3/sec is determined. The nominal sewer size of 600mm (24 inch) diameter pipe is assumed for carrying the sewer and the minimum slope of 0.08m/100m will be maintained for sewer lines.

Three type of pipelines such as sand surround, concrete/gravel surround and concrete/gravel encasement are selected for this study and the cross section design of them are given in Figure 6.4. The standard pipeline cross section is shown in Figure 6.5. The length of pipeline and its distance from existing canal boundary is given in Annexure IV.

Figure 6.4. Type of Sewer Pipes

The selection of pipe on location wise is given in Table 6.10.

Table 6.10. Location wise Sewer Line

| S.No | Sewer | Location |
| --- | --- | --- |
| 1 | Sand surround | - Akkulam boat club – Akkulam bye pass bridge |
| 2 | Concrete or gravel surround | - Akkulam bye pass bridge – Vempalavattom - Moonattumukku – Kovalam |
| 3 | Concrete or gravel encasement | - Vempalavattom – Moonattumukku bridge |

Figure 6.5. Standard Pipeline Cross Section

## To maintain the flow rate of the sewer line, pump house at appropriate locations are also identified. The major function of the pump station will maintain the pressure and the flow rate. The typical design of the pumping station is shown in Figure 6.6. The identified pumping station locations are given in Table 6.11.

## Figure 6.6. Typical Cross Section of Pumping Station

## To maintain the flow rate of the sewer line, pump house at appropriate locations are also identified. The major function of the pump station will maintain the pressure and the flow rate. The typical design of the pumping station is shown in Figure 6.7.

Table 6.11 Location of Pumping Station

| S.No | Location |  |  | Bank |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | Easting (m) | Northing (m) |  |  |  |  |
| 1 | 716768.9505 | 930240.9463 | Right |  | North of Taj Vivanta boundary |  |
| 2 | 715770.5249 | 931773.4174 | Left |  | Near Poonthura Pozhi |  |
| 3 | 715633.1250 | 932100.0712 | Right |  | Near Beach&Lake resort |  |
| 4 | 714339.9782 | 934310.7983 | Left |  | SM Lock |  |
| 5 | 713339.3209 | 935850.8168 | Left |  | Muttathara iron bridge |  |
| 6 | 713362.8405 | 935861.3159 | Right |  | Muttathara iron bridge |  |
| 7 | 712879.5899 | 936618.9447 | Right |  | Ponnara bridge |  |
| 8 | 712517.1699 | 937765.5608 | Left |  | Vallakadavu boat yard |  |
| 9 | 711465.5329 | 939111.2778 | Left |  | Chackkai road bridge |  |
| 10 | 711488.2239 | 939134.0258 | Right |  | Chackkai road bridge |  |
| 11 | 709493.2820 | 941084.7176 | Right |  | Karikkakom bridge |  |
| 12 | 708352.1036 | 942361.3265 | Left |  | Near Akkulam bye pass bridge |  |

The settlers are intensive between Vempalavattom to Panathura and the typical sewer line for house hold drainage system is designed and Figure 6.7 & 6.8 represents the house hold connection as well as the drainage systems.

## Figure 6.7 .Plan of House Connection Branch

Figure 6.8 House Hold Drainage System

The manhole or access chamber at regular interval is also provided to clean or repairing the sewer pipelines. The uniform slope between manholes has to be maintained and the typical section of access chamber is shown in Figure 6.9. The manhole at Kovalam – Muttathara STP is 35 and between Akkulam and Muttathara it is identified as 46. The location wise details of access chamber is given in Table 6.12 & 6.13.

Table 6.12 Details of Access Chamber Kovalam-Muttathara STP

| S.NO | SECTION | LEFT BANK | RIGHT BANK | TOTAL |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Kovalam – Panathura Temple | 0 m | 0 m | 10 |  |
|  |  | 495 m | 600 m |  |  |
|  |  | 935 m | 1085 m |  |  |
|  |  | 1445 m | 1500 m |  |  |
|  |  | 1755 m | 1955 m |  |  |
| 2 | Panathura Temple - Poonthura | 500 m | 545 m | 4 |  |
|  |  | 1185 m | 1295 m |  |  |
| 3 | Poonthura – Moonattumukku | 550 m | 475 m | 8 |  |
|  |  | 1480 m | 975 m |  |  |
|  |  | 1990 m | 1875 m |  |  |
|  |  | 2300 m | 2275 m |  |  |
| 4 | Moonattumukku – S M Lock | 465 m | 410 m | 4 |  |
|  |  | 690 m | 720 m |  |  |
| 5 | S M Lock – Muttathara Iron Bridge | 140 m | 350 m | 9 |  |
|  |  | 940 m | 1250 m |  |  |
|  |  | 1540 m | 1875 m |  |  |
|  |  | 1780 m |  |  |  |
| Total |  |  |  |  | 35 |

Table 6.13 Details of Access Chamber Akkulam Boat Club-Muttathara STP

| S.NO | SECTION | LEFT BANK | RIGHT BANK | TOTAL |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Akkulam boat club – Akkulam Bridge | 0 m | 0 m | 6 |  |
|  |  | 600 m | 500 m |  |  |
|  |  | 890 m | 900 m |  |  |
| 2 | Akkulam Bridge - Vempalavattom | 420 m | 400 m | 6 |  |
|  |  | 1000 m | 1020 m |  |  |
|  |  | 1170 m | 1170 m |  |  |
| 3 | Vempalavattom – Chackai Rly Bridge | 370 m | 450 m | 6 |  |
|  |  | 1170 m | 1400 m |  |  |
|  |  | 1770 m | 1700 m |  |  |
| 4 | Chackai Rly Bridge-Chackai Road Bridge | 300 m | 200 m | 4 |  |
|  |  | 750 m | 670 m |  |  |
| 5 | Chackai Road Bridge – Vallakadavu Bridge | 450 m | 850 m | 8 |  |
|  |  | 1050 m | 1550 m |  |  |
|  |  | 1550 m | 1850 m |  |  |
|  |  | 2000 m |  |  |  |
|  |  | 2300 m |  |  |  |
| 6 | Vallakadavu Bridge – Ponnara Bridge | 500 m | 425 m | 9 |  |
|  |  | 995 m | 1065 m |  |  |
|  |  | 1575 m | 1565 m |  |  |
|  |  | 2070 m | 1920 m |  |  |
|  |  | 2410 m |  |  |  |
| 7 | Ponnara Bridge – Muttathara Iron Bridge | 380 m | 515 m | 7 |  |
|  |  | 930 m | 1140 m |  |  |
|  |  | 1540 m | 1535 m |  |  |
|  |  | 1990 m |  |  |  |
| Total |  |  |  |  | 46 |

## Figure 6.9. Typical Section of Access Chamber

## 6.11  Solid Waste

Other major environmental issue in this area is solid waste disposal. The generation of solid waste include decayed vegetables and fruits, leaves, kitchen wastes, paper pieces, cloth pieces etc. At present the settlers are directly disposing the solid waste into the canal. This dumping of waste is becoming the huge barrier to flow in canal and also affecting the natural beauty of the canal.

## 6.11.1 Management of Solid Waste

Basically there are three types of disposal techniques practical in solid waste management.

- Sanitary land fill

- Composting

- Incineration

Disposal of solid waste on land is by far the most common method in use today. Land disposal in the form of a sanitary land fill has proved to be the most economical and acceptable method for the disposal of solid wastes.

Composting is biological process. Just as the micro organism in water take up bio degradable organic matter as their food, similarly microorganism in solid also use bio-degradable organic matter as food. This process can be either aerobic or anaerobic.

Incineration is nothing but the burning of solid waste. But it will produce considerable smoke and an pollution issue.

Based on the characteristics of solid wastes the appropriate disposal method is listed  out in Table 6.14 by location wise.

Table 6.14:  Solid Waste Disposal Method

| Sl. No. | Chainage (km) | Location | Disposal Method |
| --- | --- | --- | --- |
| 1. | 3.2 | Near Beach & Lake resort | Composting |
| 2. | 5.3 | Edayar Island | Composting/Land filling |
| 3. | 8.5 | Muttathara | Composting/Land filling |
| 4. | 14.9 | World Vegetable Market | Composting |

Involvement of Trivandrum Development Authority and Local NGO’s are required for effective waste management of the area.

6.12	Water Sample Results

The water samples were collected at 9 locations along the canal. The geo coordinates of the sampling locations are collected and is shown in Table 6.15.

Table 6.15: Details of water sampling stations

| Sample No | Sampling Stations | Coordinates |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | Latitude | Longitude |  |  |
| 1 | Akkulam boat club (200m towards Akkulam bridge) | 8˚31'25.6" | 76˚54'06.4" | Black coloured, weed growth |  |
| 2 | Nearby Chakka road bridge | 8˚29'29.6" | 76˚55'15.7" | Black coloured water |  |
| 3 | Muttathara pipeline bridge 1 | 8˚27'56.2" | 76˚56'05.9" | Towards Muttathara CETP |  |
| 4 | Muttathara pipeline bridge 2 |  |  | Black coloured water, weed growth |  |
| 5 | SM lock road bridge | 8˚27'53.2" | 76˚56'8.7" | Most polluted water |  |
| 6 | Beach & Lake tourist resort | 8˚26'51.2" | 76˚56'49.2" | Navigable |  |
| 7 | Nearby Temple (100m towards Thiruvallom) | 8˚25'32.1" | 76˚57'34.2" | Near Panathura temple, coir retting |  |
| 8 | Nearby Temple (100m towards Kovalam) | 8˚24'46.1" | 76˚58'00.1" | Near Panathura mosque |  |
| 9 | Nearby Quarry Site(Kovalam) | 8˚24'23.0" | 76˚58'14.8" | Coir retting |  |

The physical, chemical and biological parameters were analysed and the summarised result of the water sample analysis is given in Table 6.16.

Table 6.16: Results of water sample analysis

| SAMPLE NO/PARAMETER | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEMPERATURE | 29.4 | 29.5 | 29.6 | 29.9 | 29.5 | 29.5 | 29.7 | 29.4 | 29.6 |
| PH | 7.03 | 6.76 | 6.59 | 6.6 | 6.86 | 6.87 | 6.99 | 7 | 7.1 |
| CONDUCTIVITY (mS/cm) | 8376 | 530 | 563.7 | 525 | 497 | 526 | 1834 | 1819 | 2613 |
| TDS(PPM) | 9356 | 685 | 748 | 683 | 718 | 654 | 1906 | 1968 | 3215 |
| TURBIDITY(NTU) | 643 | 263 | 106 | 154 | 112.6 | 98.4 | 140.2 | 151 | 559 |
| TOTAL HARDNESS(mg/L) | 3750 | 125 | 15 | 60 | 70 | 65 | 750 | 1000 | 1250 |
| Ca(mg/L) | 701.4 | 40 | 7.02 | 20.04 | 14.02 | 12 | 200.4 | 150.3 | 400.8 |
| Mg(mg/l) | 486.2 | 6.08 | 6.08 | 2.43 | 8.51 | 8.51 | 60.7 | 160 | 60.7 |
| CHLORIDE(mg/l) | 4189 | 70.4 | 105.6 | 35.2 | 70.4 | 70.4 | 633 | 528 | 1002 |
| ALKALINITY (mg/l) | 70 | 122 | 102 | 102 | 96 | 80 | 74 | 72 | 86 |
| IRON(mg/l) | 0.132 | 0.35 | 0.37 | 1.20 | 0.18 | 0.08 | 0.08 | 0.11 | 0.1 |
| SULPHITE(mg/l) | 407.4 | 34.0 | 45.14 | 34.3 | 33.0 | 40.3 | 227 | 227 | 275 |
| NITRITE(mg/l) | 243.6 | 66.7 | 26.16 | 19.0 | 57.2 | 71.4 | 61.0 | 304.0 | 497.0 |
| E-coli(CFUml-1) | 260 | 26500 | 29000 | 42000 | 14000 | 1420 | 90 | 360 | 30 |

The major findings of the water sample analysis are discussed as follows;

- The temperature of the water samples observed in the range of 29.3 - 29.9˚C. The maximum was observed at Muttathara Pipeline bridge. The temperature at all the samples shows well within the permissible limit of Central Pollution Control Board (CPCB). In the case of PH the minimum value was observed at near  Muttathara (Residential).

- Conductivity of sample indicates the presence of Ca, Mg, etc.,. The higher value of conductivity was obtained for the sample collected near the sea mouth i.e Beach and Lake Tourist Resort. The higher Total Dissolved Solids (TDS) value at sample location 1 shows that, the coir waste as well as the debris were deposited in the canal banks. The value of Turbidity is high near quarry site due to the presence of large amount of suspended matter such as clay, silt or some other finely divided organic matter present in water.

- Hardness is higher for the sample 1, collected along the sea due to the presence of Sulphates, Chlorides and Nitrates of Calcium or Magnesium. The disposal of waste from this resort is also the cause of increase in hardness.

- Calcium content is high near sea mouth due to the presence of quarry, sea shells etc. Magnesium content is high near sea mouth due to the presence of quarry, sea shells etc Chloride content is higher for the sample collected along the sea due to leaching of marine sedimentary deposits, pollution from sea water, brine or industrial, domestic wastes etc.

- Higher value of Alkalinity is observed near quarry site due to the higher value of Ca, Mg etc. Higher value of Sulphite is obtained near Beach and Lake Tourist Resort. Iron content of all water samples tested is within permissible limits as per tolerance limits for surface water. The maximum iron content was observed at Sample No:4 may be due to the city drainage i.e the sewer lines are connected at this station.

- The amount of Nitrite should be nil in water. Nitrites indicates the presence of partly decomposed organic matter. E-coli value is higher for the sample collected near Muttathara pipeline bridge, SM Lock road bridge and Chakkai bridge indicating pollution caused by human or animal faeces.

To improve the water quality of the canal, the sewage pipeline from Akkulam to Muttathara and Kovalam to Muttathara has to be provided. So that, the sewage which generated from the settlers can be transported to Muttathara sewage treatment plant.

6.13	Navigation Aids

Channel markers, information, warning and direction signboards are proposed in the study stretch. Information boards should show the adjacent places with distance. At road bridges these boards can give the details of the nearby places via. road also. Warning signs should show the headway restrictions under bridges, narrow canal, depth constraints, feeder canal crossings etc. Channel markers are proposed for marking the fairway passage in wider reaches. Proposed locations for installing the sinages are given in Table 6.17.

Table 6.17: Location of Signages and Markings

| Sl.No | Place | Item |
| --- | --- | --- |
| 1 | Kovalam Left Bank | Direction, place, distance & information |
| 2 | Beach and Lake resort | Direction, place, distance & information |
| 3 | Thiruvallom | Direction, place & distance |
| 4 | Edayar | Direction, place, distance & information |
| 5 | Vallakadavu | Direction, place & distance |
| 6 | Muttathara | Direction, place, distance & information |
| 7 | Air port bridge | Direction, place & distance |
| 8 | Karikkakam Temple | Direction, place & distance |
| 9 | Akkulam bypass | Direction, place & distance |
| 10 | Akkulam Boat club | Direction, place, distance & information |

Both the sides of canal, 20 Nos of fixed shore structures and 100 floating buoys are required as navigation aids.

CHAPTER VII

COST ESTIMATION

7.1	Cleaning and Desiltation

The total dredging quantity is 271903.10 cum and the unit rate of dredging prescribed by State Irrigation Department is Rs159.7 and the estimated amount for dredging is Rs. 434.23 lakhs. The breakup of the cost is described section wise and shown in Table 7.1.

Table 7.1: Section wise break-up of the cost

| Sl.No. | Section | Dredging Quantity (m3) | Amount in lakhs |
| --- | --- | --- | --- |
| 1 | Kovalam – Panathura Temple | 19569.47 | 31.25 |
| 2 | Panathura Temple – Poonthura Pozhi | 33011.80 | 52.72 |
| 3 | Poonthura Pozhi - Moonattumukku | 45918.96 | 73.33 |
| 4 | Moonattumukku – SM Lock | 13182.41 | 21.05 |
| 5 | SM Lock – Muttathara Iron Bridge | 40149.71 | 64.12 |
| 6 | Muttathara Iron Bridge - Ponnara bridge | 20769.92 | 33.17 |
| 7 | Ponnara bridge – Vallakadavu | 26878.63 | 42.93 |
| 8 | Vallakadavu - Chackai Road bridge | 29941.61 | 47.82 |
| 9 | Chackai Road bridge - Chackai Rly bridge | 7346.38 | 11.73 |
| 10 | Chackai Rly bridge – Vempalavattom | 29633.15 | 47.32 |
| 11 | Vempalavattom - Akkulam bridge | 2058.58 | 3.29 |
| 12 | Akkulam bridge – Akkulam boat club | 3442.48 | 5.50 |
| Total |  | 271903.10 | 434.23 |

Before starting desiltation, the canal has to be cleaned by removing weeds growth, garbage and other wastes. Rs. 5 lakhs per km is assumed for cleaning of canal and the amount of Rs 92.45 lakhs is needed for cleaning activity. It is estimated that the cost of cleaning and dredging is sum of Rs.526.68 lakhs.

7.2 	Bank Protection

The total length including both the banks of 25.98 km of the canal is to be protected. The unit rate for bank protection Random rubble masonary or stone boulders strengthened with coir grids type is Rs 6024/m and the cost of bank protection is Rs. 1565.04 lakhs.

7.3 	Cross Structures

Out of 11 road bridges, 5 bridges are not following the standards like vertical and horizontal clearances. These structures have to dismantled and reconstructed. The detailed break up is given in Table 7.2. In addition to these structures one bridge is proposed near Panathura Subramania Swamy temple to connect the canal.

Table 7.2: Cost of road bridges to be reconstructed

| Sl. No. | Name of bridge | Required Length in m | Cost of bridge in lakhs | Cost of dismantling of existing bridge | Cost of approach road in lakhs | Total Cost in lakhs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Edayar Road Bridge | 50 | 500 | 23.49 | 51 | 574.49 |  |
| 2 | Ponnara Bridge | 50 | 500 | 18.01 | 65.4 | 583.41 |  |
| 3 | Puthanpalam | 50 | 500 | 17.23 | 101.4 | 618.63 |  |
| 4 | Vallakadavu bridge | 50 | 500 | 15.66 | 93.6 | 609.26 |  |
| 5 | Karikkakam Temple Road Bridge | 50 | 500 | 19.58 | 132 | 651.58 |  |
| 6 | New Bridge proposed Panathura | 50 | 500 |  | 50 | 550.00 |  |
| Total cost |  |  |  |  |  |  | 3587.37 |

The sum of Rs.3587.37 lakhs is required for dismantling, reconstruction and construction of road bridges in the study area.

Out of 7 foot bridges, 4 foot bridges at Panathura, Edayar, Karikkakam and Vazhavila are structurally weak and have to be dismantled. The foot bridges at Muttathara, Kairalinagar are to be reconstructed. The cost estimation is given in Tables 7.3 & 7.4.

Table 7.3: Cost of foot bridges for dismantling

| Sl. No. | Name of bridge | Type of bridge | No. of spans | Horizontal Clearance in m | Vertical Clearance in m | Cost of construction in lakhs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Panathura wooden footbridge | Temporary Wooden Foot Bridge | 1 | 4.5 | 2 | 1.00 |  |
| 2 | Edayar Foot Bridge | Foot Bridge, RCC | 3 | 9 | 5 | 1.50 |  |
| 3 | Vazhavila Old Foot Bridge | Foot Bridge, RCC | 3 | 7.2 | 3.8 | 1.50 |  |
| 4 | Karikkakam Vayanasala Foot Bridge | Foot Bridge, RCC | 3 | 9 | 3.74 | 1.50 |  |
| Total cost |  |  |  |  |  |  | 5.50 |

Table 7.4:   Cost of foot bridges to be reconstructed

| Sl. No. | Name of bridge | Type of bridge | No. of spans | Horizontal Clearance in m | Vertical Clearance in m | Cost of reconstruction in lakhs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Muttathara Bridge | Foot bridge | 3 | 9 | 9 | 51.5 |  |
| 2 | Karali Foot Bridge | Foot Bridge, RCC | 1 | 20 | 4.5 | 51.5 |  |
| Total cost |  |  |  |  |  |  | 103.00 |

Total cost of Rs.108.5 lakhs is needed to dismantle and reconstruct the foot bridges in the study area.

One lock and one aqueduct at Poonthura and Muttathara has to reconstructed and the cost is estimated and shown in Tables 7.5 & 7.6.

Table 7.5:  Cost of reconstruction of lock

| Sl. No. | Name of bridge | Cost of dismantling existing structure in lakhs | Cost of mechanical parts Rs. In crores | Cost of lock structure Rs. In crores | Cost of road bridge/FB Rs.in lakhs | Total cost Rs.in crores |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SM lock | 7.8 | 25 | 10 | 48 | 35.56 |  |
| Total cost |  |  |  |  |  |  | 3556.00 |

Table 7.6:  Cost of reconstruction of aqueduct

| Sl No | Name of bridge | No. of span | Horizontal Clearance | Vertical Clearance | Cost in lakhs |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Muttathara Aqueduct | 4 | - | - | 655.00 |  |
| Total cost |  |  |  |  |  | 655.00 |

To reconstruct the lock and aqueduct it is estimated that the sum of Rs.4211 lakhs is required.

7.4	Sanitation Facilities

Around 700 families are settled on the canal banks and the sanitation facilities like latrine, drainage, etc are not offered to the people. It causes serious environmental and health hazards to the people also it affects the nature of canal by discharging of the human excreta. So that, the community toilets at every 1km interval has to be constructed. The community toilet should consist of two latrines, two bath rooms, one overhead tank with pumping facility, septic tank and it should be connected with the designed drainage system. 9 community toilets between Moonattumukku and Vempalavattom has to be constructed and the cost of one community toilet is 12.5 lakhs and the total cost for constructing the community toilets is 112.5 lakhs.

7.5	Summary of Project Cost

The implementation of this project includes cleaning, dredging of canal, construction of protective measures on bank, dismantling and reconstruction of cross structures, construction of sanitation facilities, sewage pipeline and bio gas plant and deployment of navigational aids. The summary of cost is given in Table 7.7.

Table 7.7: Summary of Project Cost

| Sl. No. | Item | Quantity | Amount (in lakhs) |  |
| --- | --- | --- | --- | --- |
| 1 | Cleaning | 18.49 km | 92.45 |  |
| 2 | Dredging | 271903.10cu.m | 434.23 |  |
| 3 | Bank Protection Left Bank Right Bank | 12990 m 12990 m | 782.52 782.52 |  |
| 4 | Cross structures - Road bridges reconstruction and dismantling - Foot bridges dismantling - Foot bridges reconstruction - Lock reconstruction - Aqueduct reconstruction | 6Nos 4Nos 2 Nos 1 No 1 No | 3587.37 5.50 103.00 3556.00 655.00 |  |
| 5 | Sanitation Facilities | 9 Nos | 112.50 |  |
| 6 | Bio gas plant | 4 Nos | 1000.00 |  |
| 7 | Sewage pipeline |  | 3111.00 |  |
| 8 | Navigation aids |  | 1025.00 |  |
| Overall cost |  |  |  | 15247.09 |

The overall cost for improving the canal section between Kovalam and Akkulam section of Parvathy puthanar is Rs15247.09 lakhs. Initially, cleaning, dredging, construction of community toilets and construction of new road bridge at Panathura Subramania Swamy temple has to be taken place as Phase I for navigation. The sum of Rs.1073.13 lakhs is required to make the canal as navigable. Other activities like construction of bank protection, cross structures modification/reconstruction, sewage and bio gas plant construction and implementation of navigation aids can also be simultaneously done as Phase II.

CHAPTER VIII

CONCLUSION

The development of waterways includes several qualitative advantages like environmental upgradation, sustainable development, better mechanism of flood control, opportunities for fishing industry and promotion of domestic and international tourism activity.

The project involves the study of canal (Parvathy puthanar) from Kovalam to Akkulam. The study includes of detailed surveys for assessing physical and hydrological and environmental parameters of waterways. Detailed cost estimate for the waterway was also prepared. The following conclusions are drawn from the studies.

- The length of the waterway under the scope was 18.49km from Kovalam to Akkulam.

- The waterway was found to be stagnant in most of the section of the canal due to sewage disposal, weeds growth, garbages etc.

- Nearly 700 families are settled on the banks of canal especially the stretch between Moonattumukku and Vempalavattom and most the environmental issues occurred in this section because of poor drainage and solid waste management system.

- It is estimated that the projected wastewater generation and solid waste generation from the settlers will be 1692 lakhs litres and 6.8 lakhs kg respectively in the year of 2025. The appropriate waste management system is recommended to dispose these wastes.

- Water sample was collected from 9 locations along the study stretch. The physical, chemical and biological parameters was analysed and it was found that, the water stagnation, weeds growth and other existing wastes in the canal caused the higher levels of pollutants and this can be rectified by providing sewage pipeline and solid waste disposals.

- Narrow and shallow canals, cross structures with low clearances etc. are the other obstructions for safe navigation.

- 22 cross structures were found across the study stretch comprising of 11 road bridges,7 foot bridges,2 rail bridges and 1 lock and 1 aqueduct. Out of these 5 road bridges,2 footbridges, lock and aqueduct were found to be having low horizontal and vertical clearance and need to be reconstructed and 4 structurally weak foot bridges can be dismantled. In addition to this, one road bridge is proposed at Panathura Subramania Swamy temple to connect the canal.

- The overall cost estimation for improving this canal section is estimated as Rs15247.09 lakhs. It is also recommended that, the cleaning, dredging, construction of bridge at Panathura and community toilets as Phase I and the cost of estimation for these activities is Rs 1073.13 lakhs.

It is concluded that the development of Kovalam - Akkulam section  of Parvathy puthanaar is  technically feasible and beneficial to the society and hence, recommended for implementation at the earliest.

ANNEXURE I

SOCIO-ECONOMIC SURVEY FOR KOVALAM – AKKULAM CANAL STRETCH

- Name of Head of the Family:

- Occupation:

- Address:

House No		House Name:

Resurvey No:		Panchayat Name:				Ward No:

- Total number of family members:

i)Male			ii)Female			iii)Children

- Type of building

- House

- RCC		b) Tiled		c)if others(specify)

- Shop

- General				b) Workshop, its type & size

- Built up area

- Distance between canal and building

- Distance between canal and compound wall

- Drainage facilities:Yes/No if Yes, specify its details

- Sanitation Facilities:

- Toilet facility: Yes/ No

If NO, where do you dispose; a)over canal	b)on land

- Type of Toilet: a) Open toilet			b)Sock-pit

- Septic tank : Yes/ No

If NO, how are you disposing the human waste?

- Sources/Usage of water:

i) Corporation	ii)Well		iii)Canal	iv) Pond	v)Hand pump

a) Drinking

b) Bathing

c) Washing

d) Gardening

e) Other purposes

- Do you have 'Rain Water Harvesting' system in your building? Yes/No

- Disposal of wastewater

- Kitchen

- Bathroom

- Washing: cloths				utensils			house wash

- Others(specify)

Is a pipeline provided to septic tank for disposing the wastewater?

- Generation of Solid waste

Sources				Quantity		Type of Disposal

a) Kitchen Waste(Vegetable)

b) Paper waste

c) Glass (old lights, mirror, bottles)

d) Cloths waste

- Is there any common disposal pit for solid waste disposal?(its distance)

- Usage of canal

- If sewage pipeline is provided for wastewater disposal, are you willing to adopt the system?

- Yes

- No (Specify Reason)

- Any other remarks/comments

Interviewer Name & Signature

| ANNEXURE IV |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEFT BANK |  |  |  |  |  |  | RIGHT BANK |  |  |  |
| PIPE LENGTH(m) | DISTANCE FROM CANAL(m) | PIPE LENGTH(m) | DISTANCE FROM CANAL(m) |  |  | PIPE LENGTH(m) |  | DISTANCE FROM CANAL(m) | PIPE LENGTH(m) | DISTANCE FROM CANAL(m) |
| 0 | 5.5 | 600 | 1.5 |  |  | 0 |  | 10 | 100 | 3.5 |
| 225 | 2.75 | 40 | 0.5 |  |  | 75 |  | 7.5 | 100 | 2 |
| 90 | 2 | 300 | 2 |  |  | 100 |  | 3 | 70 | 1.5 |
| 180 | 0.5 | 500 | 4.5 |  |  | 150 |  | 2.5 | 50 | 5.5 |
| 270 | 4 | 290 | 4.5 |  |  | 275 |  | 6.5 | 100 | 4.5 |
| 170 | 1.5 | 60 | 3.5 |  |  | 275 |  | 3 | 100 | 6 |
| 340 | 10 | 180 | 7 |  |  | 125 |  | 2.5 | 350 | 1 |
| 170 | 4.5 | 180 | 6 |  |  | 20 |  | 0.4 | 500 | 2.5 |
| 210 | 1 | 180 | 5 |  |  | 65 |  | 2.5 | 450 | 2.5 |
| 50 | 1.5 | 300 | 1 |  |  | 175 |  | 2.5 | 500 | 4 |
| 50 | 0 | 100 | 1.5 |  |  | 40 |  | 1 | 470 | 1 |
| 200 | 20 | 170 | 3 |  |  | 25 |  | 0.5 | 850 | 1.5 |
| 200 | 3 | 140 | 5 |  |  | 175 |  | 21 | 600 | 1.5 |
| 140 | 7.5 | 40 | 3.5 |  |  | 150 |  | 8.5 | 100 | 2 |
| 160 | 5.5 | 70 | 4.5 |  |  | 225 |  | 2.5 | 300 | 3.5 |
| 100 | 3.5 | 120 | 3 |  |  | 200 |  | 2 | 200 | 6 |
| 110 | 7.5 | 350 | 1 |  |  | 270 |  | 15.5 | 225 | 3 |
| 140 | 2.5 | 450 | 1.5 |  |  | 100 |  | 3.5 | 340 | 3 |
| 150 | 10 | 900 | 1.5 |  |  | 75 |  | 8.5 | 160 | 1.5 |
| 60 | 3.5 | 500 | 2 |  |  | 100 |  | 2 | 140 | 11 |
| 140 | 5 | 400 | 5 |  |  | 140 |  | 1.5 | 100 | 2 |
| 130 | 1.5 | 600 | 3.5 |  |  | 150 |  | 2 | 100 | 0.5 |
| 70 | 3.5 | 500 | 1 |  |  | 300 |  | 7.5 | 200 | 0.5 |
| 350 | 3.5 | 450 | 1 |  |  | 375 |  | 5 | 100 | 0.5 |
| 250 | 1.5 | 150 | 0.2 |  |  | 85 |  | 30 | 100 | 1.5 |
| 120 | 3 | 150 | 4 |  |  | 175 |  | 4 | 40 | 2 |
| 210 | 2 | 450 | 4 |  |  | 500 |  | 7 | 55 | 2 |
| 90 | 3 | 45 | 10 |  |  | 900 |  | 16 | 100 | 2.5 |
| 240 | 3.5 | 180 | 7.5 |  |  | 400 |  | 40 | 30 | 2.5 |
| 180 | 22.5 | 400 | 16.5 |  |  | 150 |  | 6 | 60 | 2.5 |
| 180 | 5 | 120 | 15 |  |  | 90 |  | 1.5 | 30 | 5 |
| 130 | 1 | 375 | 4 |  |  | 170 |  | 0.5 | 245 | 2.5 |
| 50 | 2 | 240 | 14 |  |  | 260 |  | 6 | 20 | 2.5 |
| 90 | 1.5 | 180 | 6 |  |  | 350 |  | 0.5 | 75 | 2.5 |
| 100 | 4 | 180 | 3 |  |  | 900 |  | 1 | 115 | 2.5 |
| 225 | 2 | 150 | 18 |  |  | 625 |  | 5 | 55 | 2.5 |
| 225 | 2.5 | 400 | 7 |  |  | 500 |  | 2 | 110 | 2.5 |
| 80 | 1 | 150 | 11 |  |  | 400 |  | 1 | 285 | 5 |
| 60 | 3 | 220 | 15 |  |  | 400 |  | 3 | 235 | 7.5 |
| 800 | 8 |  |  |  |  | 350 |  | 3.5 | 160 | 7.5 |